Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

II B.TECH, I SEMISTER MID-2

BUILDING MATERIALS AND CONSTRUCTION

Objective                                                              20x1/2=10

1)   When the brick is cut along the length, it is called                                                                   [     ]

- King closer     b)  Half header       c)  Queen closer       d)  Half stretcher

2)   The specific gravity of stones lies between                                                                                [     ]

- 1.5 to 2.0     b)  2.1 to 2.3      c)  2.3 to 2.5         d)  2.5 to 2.7

3)  Gypsum has negligible                                                                                                                    [      ]

- Durability   b)  Shrinkage   c)  Chemical properties    d)  Flexibility

4)  Which one of these is not an objective of seasoning of timber                                                              [     ]

- Reduction of weight     b) Reduction of natural defects in timber   c) Reduction in shrinkage and warping  d) Increase in strength and durability

5)  Seasoning of timber is needed to                                                                                                   [     ]

- Straighten the timber  b) Soften the timber  c)  Harden the timber  d) Remove sap from the timber

6)  Which of the following wood is not appropriate for constructing bridge                            [     ]

- Nageshwar       b)  Sal              c) Babool     d) Teak

7)  Seasoning of timber is essential to                                                                                               [     ]

- Increase the vermin resistance    b) Increase the fire resistance    c) Reduce the microbial substance   d)  Expel the moisture present in timber

8)  The construction of building materials bonded together with mortar is defined as          [     ]

- Concrete    b )   Batching    c)  Curing   d)  Masonry

9)  The construction of ashlar masonry is performed using ------ stone blocks                            [     ]

- Square or rectangle       b) irregular       c) Polygonal             d) Square

10)  Before testing the cement for its setting time, the cement is tested for,                            [    ]

- Fineness     b) Soundness     c)  consistency       d) strength

11) The percent of volume that is occupied by aggregates in cement concrete is,                   [   ]

a)  70-80%      b) 60-70%    c)  50-60%   d) 40-50%

12)  The cement which should not be used with any admixtures is,                                       [    ]                                                         
        a) Rapid hardening cement         b) High alumina cement

c) High strength cement              d) Super sulphated cement

13)  The compaction factor test is carried out for determining its,                                              [    ]
          a) porosity     b) workability      c) Degree of compaction        d) strength

14)   Hyperbolic paraboloid is a type of                                                                                  [   ]                                                                        
        a) Flat roof     b) Pitched roof       c) Shell roof           d) None of the above

15)  Distemper is used for coating                                                                                                        [    ]                                                                                                          
         a) Wood work     b) Compound walls     c) Interior surfaces     d) External concrete surfaces

16)  Scaffolding is constructed for the heights of more than                                                         [    ]                                                                                                                                                                                                                                      
         a) 1 M   b) 1.5 M     c) 2.0 M      d) 2.5 M

17)  The foundations are placed below ground level to increases                                               [     ]                                    
    a) Strength         b) Stability of structure   c) workability  d)  none of these

18)   The foundation in which cantilever beam is provided to join footings is known as        [    ]

a)  Strip footing      b) Strap footing        c) combined footing       d) Raft footing

19)  The bricks used for hidden masonry works should be,                                               [    ]                                           
         a) IIIrd class    b) ISt class      c)  IInd  class      d) Any of the above

20)  The homogeneous solution of resin in alcohol is known as,                                           [    ]
         a) paint         b) tarpentine         c) varnish      d) pigment

Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

II B.TECH, I SEMISTER MID-2

BUILDING MATERIALS AND CONSTRUCTION

Write any three questions                                                                                                                           3x10=30

Paper set by Sk. Mizba nureen

1) (a)  Define damp proof course and write the applications for this in construction?                                                                             (b)  Write the effects of damp proof?                                                                                                                                              2)  (a)  Write the manufacturing process of common and encaustic tiles?                                                                     (b)  Write the uses of tiles?                                                                                                                                                          3) (a)  Describe the constituents of paints?                                                                                                                        (b)  Write a short notes on the paints on old wood?                                                                                                        4)  What is a lintel? Write the different types of lintels?                                                                                                    5)  Write the different types of stair cases with neat sketches?