Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL ENGINEERING

II B.TECH, I SEMISTER MID-1

FLUID MECHANICS

NAME:                                                 Objective                                                       20×1/2=10    Roll no:

1) Specific volume is reciprocal of:                                                                                                    [     ]

- Mass      b) volume        c) compressibility           d) density

2) 1 poise:                                                                                                                                                [    ]

- 1/10Ns/m2     b) 1/100Nsec/m2      c) 1/10Ns/m        d) 1/1000Ns/m2

3) Mercury is most commonly used in barometers because it has:                                           [    ]

- Low vapour pressure       c)constant vapour pressure

- More vapour pressure   d)None of the above

4) The pressure above the atmospheric pressure is known as:                                                   [    ]

- Vacuum pressure            c) Gauge pressure

- Absolute pressure   d) Absolute zero pressure

5)  Piezometer is used for measuring:                                                                                                [    ]

- Vacuum pressure           c) Gauge pressure

- Atmospheric pressure   d)Barometric pressure

6)  Reynolds number for turbulent flow:                                                                                            [   ]

a) <2000      b) >2000      c) <4000    d) >4000                                                                                     7) A flow is said to be laminar flow when its value is:                                                                                    [    ]

a) Less than Re       c) More than Re

b) Equal to Re         d) None of the above

8) Equation of continuity is based on the principle of:                                                                    [    ]

- Conservation of energy     c) Conservation of momentum

- Conservation of mass        d) None of the above

9) Bernoulli’s equation is applied for:                                                                                                  [   ]

a) Steady flow      b) Continuous flow    c) Both (a)&(b)     d) None of the above

10) Momentum equation is based on the principle of:                                                                    [   ]

a) Conservation of mass   b) Conservation of energy  c) Conservation of momentum d) a&b

11) Moment of momentum equation is given by:                                                                            [    ]

a)T=PQ[v1r2-v2r1]   b)T=PQ[v2r2-v1r1]   c)T=PQ[v1r2-v2r1]   d)T=PQ[v1r1-v2r2]

12)  Liquids are:                                                                                                                                        [    ]
        a) Compressible        b) In compressible     c) (a)&(b)      d) None of the above

13) A fluid which does not have viscosity surface tension and is incompressible in nature is known as:                                                                                                                                                 [    ]
          a) Real fluid   b) Ideal fluid      c) Newtonian fluid     d) All of the above

14) Units for specific volume:                                                                                                [   ]
        a) N/m2     b) N/m    c) m3/kg       d) Ns/m2

15) Which type of flow the fluid particles moves in a zigzag manner:                                         [   ]                                                                               
         a) Laminar flow  b) Turbulent flow c) Uniform flow   d) Non uniform flow

16) Poise is a unit of:                                                                                                                              [   ]
         a) Viscosity   b) Capillarity c) Surface tension   d) Vapor pressure

17) The addition of potential head and pressure head is known as:                                           [    ]                                                                                     
         a) Piezometric head b) Differential manometric head c) kinetic head   d) None of the above

18) Stream line path line streak line is the identical when the flow is:                                       [   ]
         a) Steady flow   b) unsteady flow     c) Potential flow   d) None of the above

19) A laminar flow can also be called as:                                                                                            [   ] 
         a) Viscous flow b) Potential flow   c) Non viscous flow d) steady flow

20) A real fluid has the following property:                                                                                        [   ]
         a) Compressibility        b) Capillarity     c) Viscosity       d) None of the above

Gokula Krishna college of Engineering –Sullurpet

Department of  CIVIL ENGINEERING

II B.TECH, I SEMISTER MID-1

FLUID MECHANICS

Write any three questions 	                                                                                                   3×10=30

1)  Briefly   discuss   about differential manometers with neat sketch?

2)  (a)  Differentiate:                                                                                                                                                                                                    		     (i) Uniform flow & Non uniform flow    ii)Laminar flow & Turbulent flow                                                                                                                                                                                                                                                                                                                                                                                                                 		      (iii)  real fluid & ideal fluid                      iv)Newtonian & Non Newtonian fluids

(b) Derive  surface  tension ?Prove that the relationship between surface tension and pressure inside a droplet                             	of  liquid   in excess of  outside pressure is  given by   p=4σ/d  ?

3)  (a) Derive  an expression for continuity equation  for  3-Dimensional flow?

(b) Derive the velocity potential function and stream function and mention justify the properties?

4) (a) Derive an expression for force exerted on a pipe bend?

(b) Derive an expression for Euler s equation of motion?

5)   (a) Derive the inclined plane surface submerged in a liquid with neat sketch?

(b) What is buoyancy, center of buoyancy, buoyant force, meta centric height and stability of      	submerged bodies with neat diagram?

Paper set by T. Mounika