GOKULA KRISHNA COLLAGE OF ENGINEERING-SULLUTEST

DEPARTMENT OF CIVIL ENGINEERING

II B. TECH, I SEMISTER, II MID

SURVEYING- I

Name of the student:                                                                                     Date   :12-11-2015

Roll no:                                                                                                            sign. Of inviglator     :

OBJECTIVE TYPE EXAMINATION                                                                       Time: 20 min

Answer the following questions:

1. A theodolite in which the telescope can be revolved through a complete revolution in a

Vertical plane Is known as                                                                                                                          (        )

(A)Non transit theodolite (B) Tilting theodolite (C) transit theodolite (D) All

2. The face left portion is also called                                                                                                             (        )

(A) Telescope inverted (B) Telescope normal  (C) Telescope reversed  (D)None

3. If N is the number of sides of a traverse, the sum of measured exterior angles should be equal to(      )

(A) (2n-4)x 90   (B) (2n+4)x90   (C) (n+4)x90    (D) (n-4)x90

4. In a closed traverse the algebraic sum of latitude and departure must be equal to                           (       )

(A) 0                  (B)1            (C) 2              (D)  5

5. Balance of traverse is done according to the                                                                                              (       )

(A)Transit rule  (B) Prismoidal rule   (C)Trapezoidal rule  (D) Mid ordinate rule

6.The datum adopted in INDIA is the                                                                                                                (       )

(A) MSL at Madras     (B) MSL at Bombay  (C) MSL at karachi   (D) none

7. A 30m chain is divided into                                                                                                                             (       )

(A) 100 links  (B) 150 links    (C) 300 links     (D) 400 links

8.The road features of a contour is indicated by the ______ colour                                                           (       )

(A).Blue          (B)Red    (C)Black   (D)Brown

9.The closing error in a closed traverse is adjusted by the                                                                            (      )

(A) Lehmannˈs rule   (B) Bowditchˈ rule   (C) Slide rule   (D)All

10. Inaccessible points may be located by the                                                                                                 (      )

(A) Resection method   (B) Intersection method    (C) Radiation method   (D) Mechanical method

11. The least count of the vernier                                                                                                                    (       )

(A) 0° 15     (B) 2 °                   (C)0° 10                   (D) 0° 30

12. The vertical circle in a theodolite is used for measuring________                                                       (       )

(A) Vertical angles   (B) Horizontal angles    (C) Included angles   (D) Exterior angles

13.The foot screws are used for                                                                                                                         (       )

(A) Taken the bubble to center  (B) Centering  (C) Levelling      (D) All

14. The line ranger is used for                                                                                                                             (      )

(A) Ranging                    (B)  Centering                      (C) Levelling      (D) All

15. The vertical distance between any two contours is                                                                                  (      )

(A) Contour gradient (B) Contour interval    (C) Contour interpolation   (D) None

16. Hand level is used for_____________ survey                                                                                             (     )

(A) Reconnaissance           (B) Preliminary       (C) Both A & B          (D )None

17.The length measured parallel to meridian  is                                                                                               (     )

(A) Latitude        (B) Departure                (C) Bearing             (D) Angle

18. The vertical circle is either graduated from                                                                                              (        )

(A) 0° to 90°               (B) 90° to 180 °            (C) 0 °to 180°             (D) 0° to 360°

19. The sum of interior angles should be equal to                                                                                           (      )

(A) (2n-4)x90°           (B) (2n+4)x90°              (C) (n-4)x90°               (D) (n+4)x90°

20. Least count of leveling staff                                                                                                                           (      )

(A) 0.5m                     (B) 0.05m                       (c) 0.005m                  (D) 0.0005m

GOKULA KRISHNA COLLAGE OF ENGINEERING-SULLUTEST

DEPARTMENT OF CIVIL ENGINEERING

II B. TECH, I SEMISTER, II MID

SURVEYING- I

-----------------------------------------------------------------------------------------------------------------------Answer   any three questions:

1.  (a). The following staff readings are observed successively, with a level .The instrument having                                                                         .                been moved after 3rd , 6th , 8th  readings: 2.228; 1.606; 0.998; 2.090; 2.864; 1.262; 0.602; 1.982; 1.044; 2.684m. Enter the above readings in a page of field book and calculate the R.L. of points,                 By the application of  (1) H.I. method   (2) RISE AND FALL method.                                                                      (b). Define the following terms: (1) Face right observation (2) Telescope normal.

2.   (a) . Explain characteristics of contour.   (b).  Explain uses of contour maps.                                               (c).  Explain interpolation of contour by arithmetic calculations.

3.      (a).  Explain reciprocal leveling.      ( b). Explain measurements of horizontal angels by repetition method.  (c) . Define the following terms (1)face left observations (2) Changing face.

4.     (a) .  The table below gives the lengths and bearings of the lines of a traverse ABCDE,the length and Bearing of EA having been omitted. Calculate the length and bearing of the line EA.

| LINE | LENGTH(m) | bearing |
| --- | --- | --- |
| AB | 204 | 87° 30ˈ |
| BC | 226 | 20° 20ˈ |
| CD | 187 | 280° 0ˈ |
| DE | 192 | 210° 3ˈ |
| EA | ___ | ___ |

(b). Explain curvature and refraction.

5.      (a).  The perpendicular offsets were taken  at 10m intervals from a survey line  to an regular     Boundary line:   3.25 , 5.60,4.20,6.65,8.75,6.20,3.25,4.20,5.65m.Calculate the area enclosed  Between the survey line the regular boundary line, application of  (1) Aveage ordinate rule (2).Trapezoidal rule   (3) simpsonˈs rule.

(b).  Write a short notes on hand level and foot rule clinometers.

PAPER SET BY G.SUREKHA