Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

II B.TECH, I SEMISTER MID-1

STRENGTH OF MATERIALS

Objective                                                              20x1/2=10

1) Stress is                                                                                                                                               [    ]

- External force      b) Internal resistive force         c) Axial force            d) Radial force

2) Which of the following is not a basic type of strain?                                                                [    ]

- Compressive strain     b) Shear strain      c) Area strain        d) Volume strain

3) Tensile Strain is                                                                                                                                  [    ]

- Increase in length / original length    c) Decrease in length / original length

- Change in volume / original volume   d) All of the above

4) Compressive Strain is                                                                                                                        [    ]

- Increase in length / original length       c) Decrease in length / original length

- Change in volume / original volume      d) All of the above

5) Volumetric Strain is                                                                                                                            [    ]

- Increase in length / original length       c) Decrease in length / original length

- Change in volume / original volume      d) All of the above

6) Hooke’s law is applicable within                                                                                                      [    ]

- Elastic limit      b) Plastic limit      c) Fracture point      d) Ultimate strength

7) Young’s Modulus of elasticity is                                                                                                       [    ]

- Tensile stress / Tensile strain       c) Shear stress / Shear strain

- Tensile stress / Shear strain         d) Shear stress / Tensile strain

8) Poisson’s ratio is                                                                                                                                  [    ]

- Lateral strain / Longitudinal strain     c) Shear strain / Lateral strain

- Longitudinal strain / Lateral strain      d) Lateral strain / Volumetric strain

9) A rod, 120cm long and of diameter 3.0 cm is subjected to an axial pull of 18 KN.

The stress in ------------------- N/mm2.                                                                                                   [    ]

- 22.57         b) 23.47       c) 24.57             d) 25.47

10) The relationship between Young’s modulus (E), Bulk modulus (K) and Poisson’s ratio (µ) is given by                                                                                                                                                      [    ]

- E=2K(1-2µ)     b) E=3K(1-2µ)    c) E=2K(1-2µ)       d) E=2K(1-3µ)

11) The relationship between Young’s modulus (E), Modulus of rigidity (C) and Bulk modulus (K) is given by                                                                                                                                            [    ]

- E=9CK/(C+3K)       b) E=9CK/(2C+3K)     c) E=9CK/(3C+K)    d) E=9CK/(C-3K)

12) At the points of shear force changes sign, bending moments will be                                   [    ]
        a) Maximum            b) Minimum         c) Zero              d) None

13) At the points of bending moment changes sign, shear force will be                                     [    ]
          a) Maximum   b) Minimum      c) Zero        d) None

14) Which moment is considered as positive                                                                        [   ]
        a) Hogging      b) Sagging       c) Clockwise            d) None

15) Maximum bending moment in a Simply Supported beam having a concentrated load at the centre will be                                                                                                                                      [   ]
         a) WL        b) WL/2        c) WL/4           d) None

16) Maximum bending moment in a Simply Supported beam having a UDL over entire length will be                                                                                                                                                         [    ]
         a) wL2/2   b) wL2/4     c) wL2/8       d) None

17) At the supports of a simply supported beam, bending moment will be                               [   ]
         a) Maximum    b) Minimum      c) Zero        d) None

18) At the supports of a simply supported beam, shear forces will be                                         [   ]
         a) Maximum    b) Minimum      c) Zero        d) None

19) In case of a cantilever beam, shear force at the fixed end will be                                          [   ]
         a) Maximum    b) Minimum      c) Zero        d) None

20) In case of a cantilever beam having UDL, bending moment variation will be                      [   ]
         a) Linear         b) Parabolic         c) Cubic       d) None

Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

II B.TECH, I SEMISTER MID-1

STRENGTH OF MATERIALS

Write any three questions                                                                                                                           3x10=30

1) a) Explain and draw stress & strain diagram for mild steel?

b) A tensile test was conducted on mild steel bar. The following data was obtained from the test: diameter of the steel bar=3cm,gauge length of the bar=20cm,load at elastic limit=250kn, extension at a load of 150kn=0.21mm, maximum load=380kn, total extension =60mm,diameter of the rod at the failure=2.25cm.  Determine, the young’s modulus, stress at elastic limit, % elongation, % decrease in area.

2)  Two brass rods and one steel rod together support a load as shown in fig. if the stress in brass and steel are not to exceed 60n/mm2 and 120n/mm2, find the safe load that can be supported. Take E for steel=2x105 n/mm2 and foe brass=1x105n/mm2, the cross-sectional area of steel rod is 1500mm2 and of each brass road is 1000mm2.

3) Derive shear force and bending moment diagram for a simply supported beam carrying a uniformly varying load for zero at each end to w per unit length at the centre.

4) A simply supported beam of length 5m carries a uniformly increasing load of 800N/m run at one end to 1600N/m run at other end. Draw the shear force and B.M diagram for the beam also calculate the position and magnitude of maximum bending moment.

5) Draw the S.F and B.M diagram of a simply supported beam of length 7m, carrying uniformly distributed loads as shown in fig.

Paper set by K.Deepak Kumar