GOKULA KRISHNA COLLEGE OF ENGINEERING-SULLURPET

CIVIL ENGINEERING DEPARTMENT

II B.TECH  I SEMISTER I MID EXAM

SURVEYING-OBJECTIVE TYPE QUESTIONS

Time: 20min.                                    ROLL NO:                              Name of the student:

Signature of invigilator:

OBJECTIVE

1.The length of the chain is measured from                                                                                                  (        )

a) Centre of one handle to centre of other handle    b) Outside of one handle to outside of other   handle   c)  Outside of one handle to inside of other handle  d) Inside of one handle to inside of other handle

2.  Which of the following methods of offsets involves less measurement on ground                        (        )

a) Method of perpendicular offsets               b) Method of oblique offsets

c) Method of ties                                               d) All involve equal measurement on ground

3. The permissible error in chaining for measurement with chain on rough or hilly ground is           (       )

a) 1in 100                           b) 1 in 500                  c) 1 in 250               d) 1 in 1000

4. Alidade is used in                                                                 	       (       )

a) Tachometry          b) plane tabling              c) Chaining                 d) Compass surveying

5 .Advantage of working from whole to part                                                                                                 (       )

a) Take less time         b) Plotting is easy          c) Errors are reduced           d) Need less observations

6. A building is                                                                                                                                                       (       )

a) Obstacle for chaining not for ranging               b) Obstacle for ranging not for chaining

c) Obstacle for both chaining & ranging               d) Not an obstacle to chaining or ranging

7 .Cross staff is an instruments used                                         			                                  (      )

a) Measuring approximate horizontal angle         b) Set out right angles

c) Measuring bearings of lines                                 d) none of the above

8. Select the incorrect statement                                                                                                                      (       )

a) The true meridians at different places are parallel to each other   b) The true meridian at any place is not variable  c) The true meridians converge to a point in northern & southern hemispheres d) The maps are prepared by national survey departments of any country are based on true meridian

9. Which of the following is not used in measuring perpendicular offsets                                            (        )

a) Line ranger             b) Steel tape        c) Optical square         d) Cross staff

10 .The horizontal angle between true meridian and magnetic meridian at a place is called           (        )

a) Azimuth             b) Declination        c) Local attraction                        d) Magnetic bearing

11. The prismatic compass and surveyors compass                                                                                     (        ) a) Given W.C.B. of a line and Q.B.of a line                 b) Both give Q.B.of a line and W.C.B. of a line                c) Both give Q.B. of a line                                             d) both give W.C.B. of a line

12. Which of the following angles are set out with the help of French cross staff                                (        )  a) 45° only                b) 90° only            c) Either 45°or 90°                   d) Any angle

13. For a well conditioned triangle, no angle should be less than                                                            (        )  a) 30°                 b) 20°                  c) 45°              d) 60°

14.In a Q.B. System, a whole circle bearing 293°30ˈ can be expressed as,                                             (       )  a)W23°30ˈN           b)N66°30ˈW           c) S113°30ˈN             d)N23°30ˈW

15. Local attraction may exist in compass surveying due to                                                                       (       )  a) Incorrect leveling of magnetic needle              b) Loss of magnetism of needle                                                       c) Friction of the needle at the pivot                     d) Presence of magnetic substance near instruments

16. The correction for sag is always                                                                                                                   (      )   a) Always additive                                                        b) Always subtractive                                                                                  c) always zero                                                                d) Sometimes additive and sometimes subtractive

17. For a line AB                                                                                                                                                     (      )   a) The F.B.OF AB and B.B.of a line differ by 180°            b) The F.B.of AB and B.B.of BA differ by 180°                   c) Both (a) and (b) are correct                                             d) None is correct

18. Principle of compass surveying is                                                                                                               (       )

a) Triangulation    b) Traversing           c) Plane tabling          d) Tachometry

19. If the length of the chain is found to be short on testing it can be adjusted by                               (      )

a) Strengthen the links                                                       b) Removing one or more circular rings

c) Closing the joints of the rings if opened out               d) All the above

20. What are the dimensions of table used in plane tabling                                                                       (       )

a) 45X60                                                                                  b)60X75

c)115x95                                                                               d)20X30