Gokula Krishna college of Engineering –Sullurpet

Department of  EEE

II B.TECH, I SEMISTER MID-2

NAME:                                         Objective                                                   20×1/2=20               Roll no:

1) Hydraulic turbine converts :                                                                                                            [    ]

- Mechanical energy into hydraulic energy     c) Hydraulic energy  into electrical energy

- Hydraulic energy into mechanical energy     d) Electrical energy into hydraulic energy

2) The force exerted by a jet of water on a stationary vertical plate in the direction of jet is given by:                                                                                                                                                  [    ]

- ρav     b) ρav2      c) ρa2v       d) ρav3

3) Cavitations in hydraulic turbo machine is caused by  :                                                             [    ]

- Low pressure   c) Local pockets of excessive pressure

- Low velocity   d)  Region of low barometric pressure

4) The  surge tank is located in a hydro electric power station is near to :                               [    ]

- Catchment area            c) Turbine

- Reservoir                      d) Tail race

5)  A Pelton wheel is ideally suited for:                                                                                              [    ]

a) Low head low discharge         b) High head low discharge

c) High head high discharge       d ) Low head high discharge

6)   The value of speed ratio for Kaplan turbine is:                                                                          [    ]

a) 0.5   b) 1.0      c) 1.5   d) 2.0

7)    Muschel curves means curves at:                                                                                                [    ]

a) Constant efficiency        c) Constant speed

b)  Constant head               d) Constant discharge

8)   The speed ratio of hydraulic turbine is proportional to:                                                            [    ]

a)1/D          b)N2      c) 1/ (H)1/2       d) H

9)   Series operation of centrifugal pumps results in:                                                                       [    ]                                                                                                                                                                                                                                            .      a) High head    b) High discharge

c) Low speed operation     d) Reduced power consumption

10)  Cavitations in centrifugal pump can be reduced by:                                                                [   ]

a) Reducing the discharge   b) Throttling the discharge                                                                                         c) Reducing the suction head    d)  Increasing the velocity

11)  In a centrifugal pump, the regulating valve is provided on:                                                    [   ]

a) The suction pipe   b) Delivery pipe   c) Casing   d) None

12)  A centrifugal pump starts delivering liquid only when the pressure rise in the impeller is equal to the:                                                                                                                                              [    ]
        a) Static head        b) Kinetic head    c) Velocity head     d) Manometric head

13)  Which among are the  elements of hydro electric power station:                                      [     ]                                                                        
          a) Catchment area ad  water reservoir   b) Dam and intake   c) Tail race  d)  All

14)   Francis turbine is a ----------flow turbine:                                                                      [     ]            
        a) Mixed     b) Radial      c) Out ward       d) Inward

15) For the same  power output the operating charges are minimum in:                                  [   ]                                                    
         a)Thermal plant b) Gas turbine c) Hydel plant   d) Nuclear plant

16) The NPSH of a centrifugal pump is equal to:                                                                              [    ]
         a) Slip b) Discharge   c) Average velocity   d) Velocity at a point

17) A turbine is a--------- machine:                                                                                                        [   ]
         a) Electrical   b) Mechanical    c) Hydraulic      d) None

18) A pelton wheel turbine is a--------------- turbine                                                                           [   ]
         a) Impulse    b) Reaction    c) Axial   d) None of the above

19) The directio0n of flow of water in pelton turbine is:                                                                [   ]
         a) Radial  b) Axial   c) Vertical  d) None of the above

20)  The efficiency of a turbine is denoted by:                                                                                  [   ]
         a)η       b) ρ    c) е  d) None of the above

Gokula Krishna college of Engineering –Sullurpet

Department of  EEE

II B.TECH, I SEMISTER MID-2

FM&HM

Write any three questions 	                                                                                                   3×10=30

1) Define hydraulic turbine and explain about different heads and efficiencies that are associated with          turbines ?

2)  (a) Explain pelton wheel:

(b) A pelton wheel working under a head of 52m develop a shaft power of 92KW at a speed of r.p.m if the         overall efficiency is 82.5% and cv=0.98 find the jet diameter  the diameter of the bucket circle , the size of the bucket and the number of buckets required assume the maximum efficiency condition that the peripheral velocity is 0.47 times the velocity of jet ?

3)  (a) What do you mean by priming?

(b) Explain the following multi stage centrifugal pump?

4) (a) Derive the expression for the specific speed of a turbine?

(b) Define cavitation and explain how cavitation can be avoided?

5) Derive an expression for moving inclined and vertical plates with neat sketches?

Paper set by T.Mounika