Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

II B.TECH, I SEMISTER MID-1

STRENGTH OF MATERIALS

Objective                                                              20x1/2=10

1) Stress is                                                                                                                                               [    ]

- External force      b) Internal resistive force         c) Axial force            d) Radial force

2) Which of the following is not a basic type of strain?                                                                [    ]

- Compressive strain     b) Shear strain      c) Area strain        d) Volume strain

3) Tensile Strain is                                                                                                                                  [    ]

- Increase in length / original length    c) Decrease in length / original length

- Change in volume / original volume   d) All of the above

4) Compressive Strain is                                                                                                                        [    ]

- Increase in length / original length       c) Decrease in length / original length

- Change in volume / original volume      d) All of the above

5) Volumetric Strain is                                                                                                                            [    ]

- Increase in length / original length       c) Decrease in length / original length

- Change in volume / original volume      d) All of the above

6) Hooke’s law is applicable within                                                                                                      [    ]

- Elastic limit      b) Plastic limit      c) Fracture point      d) Ultimate strength

7) Young’s Modulus of elasticity is                                                                                                       [    ]

- Tensile stress / Tensile strain       c) Shear stress / Shear strain

- Tensile stress / Shear strain         d) Shear stress / Tensile strain

8) Poisson’s ratio is                                                                                                                                  [    ]

- Lateral strain / Longitudinal strain     c) Shear strain / Lateral strain

- Longitudinal strain / Lateral strain      d) Lateral strain / Volumetric strain

9) A rod, 120cm long and of diameter 3.0 cm is subjected to an axial pull of 18 KN.

The stress in ------------------- N/mm2.                                                                                                   [    ]

- 22.57         b) 23.47       c) 24.57             d) 25.47

10) The relationship between Young’s modulus (E), Bulk modulus (K) and Poisson’s ratio (µ) is given by                                                                                                                                                      [    ]

- E=2K(1-2µ)     b) E=3K(1-2µ)    c) E=2K(1-2µ)       d) E=2K(1-3µ)

11) The relationship between Young’s modulus (E), Modulus of rigidity (C) and Bulk modulus (K) is given by                                                                                                                                            [    ]

- E=9CK/(C+3K)       b) E=9CK/(2C+3K)     c) E=9CK/(3C+K)    d) E=9CK/(C-3K)

12) At the points of shear force changes sign, bending moments will be                                   [    ]
        a) Maximum            b) Minimum         c) Zero              d) None

13) At the points of bending moment changes sign, shear force will be                                     [    ]
          a) Maximum   b) Minimum      c) Zero        d) None

14) Which moment is considered as positive                                                                        [   ]
        a) Hogging      b) Sagging       c) Clockwise            d) None

15) Maximum bending moment in a Simply Supported beam having a concentrated load at the centre will be                                                                                                                                      [   ]
         a) WL        b) WL/2        c) WL/4           d) None

16) Maximum bending moment in a Simply Supported beam having a UDL over entire length will be                                                                                                                                                         [    ]
         a) wL2/2   b) wL2/4     c) wL2/8       d) None

17) At the supports of a simply supported beam, bending moment will be                               [   ]
         a) Maximum    b) Minimum      c) Zero        d) None

18) At the supports of a simply supported beam, shear forces will be                                         [   ]
         a) Maximum    b) Minimum      c) Zero        d) None

19) In case of a cantilever beam, shear force at the fixed end will be                                          [   ]
         a) Maximum    b) Minimum      c) Zero        d) None

20) In case of a cantilever beam having UDL, bending moment variation will be                      [   ]
         a) Linear         b) Parabolic         c) Cubic       d) None

Gokula Krishna college of Engineering –Sullurpet

Department of MECHANICAL Engineering

II B.TECH, I SEMISTER MID-1

MECHANICS OF SOLIDS

Write any three questions                                                                                                                           3x10=30

1)  a) Relationship between modulus of elasticity and modulus of rigidity?

b) A bar of steel is 60mmX60mm in section and 180mm long. It is subjected to a tensile load of 300kn along the longitudinal axis and tensile loads of 750kn and 600kn on the lateral faces. Find the change in volume. Take E=200gn/m2, and 1/m=0.3

2) A steel rod 20mm in diameter passes centrally through a steel tube of 25mm internal diameter and 30mm external diameter. The tube is 800mm long and is closed by rigid washers of negligible thickness which are fastened by nuts threaded by nuts threaded on the rod. The nuts are tightened until the compressive load on the tube is 20kn. Calculate the stresses in the tube and the rod.                                                                                                                                                   Find the increase in these stresses when one nut is tightened by one-quarter of a turn relative to the other. There are 4 threads per 10cm. take E=2x105 n/mm2.

3) A cantilever of length 5m is loaded as shown in fig. Draw the S.F and B.M diagram for the cantilever

4) Derive shear force and bending moment diagram for a simply supported beam carrying a uniformly varying load for zero at each end to w per unit length at the centre.

5) A simply supported beam of length 10m, carries the uniformly distributed load and two point loads as shown in fig.. Draw the S.F and B.M diagram for the beam. Also calculate the maximum bending moment.

Paper set by K.Deepak Kumar