Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

III B.TECH, I SEMISTER MID-1

DESIGN&DRAWING OF REINFORCED CONCRETE SRUCTURES

Objective                                           20x1/2=10

1.Concrete basically consists of	  							[           ]

a) water, cement 	b)cement, aggregate	 c)water, cement          d)water, cement, aggregate

2.As per the IS456-2000 youngs modulus of elasticity					[           ]

a)5000√fck 	b)5500√fck	 c)5000√fcy	 d)50√fck

3.The nominal mix concrete for M20 grade						[           ]

a)1:2:1 		b)1:3:6		 c)1:2:4		 d)1:1 ½:2

4.Unit weight of steel is									[            ]

a)78.5KN/m  	b)78.5KN 	c)78.5KN/m3 d)78.5KN/m2

5.Modular ratio M is									[           ]

a)360/σcbc b)280/σcbc c)240/σcbc d)120/σcbc

6.Unit weight of plain concrete is							[           ]

a)25KN/m3 b)24KN/m3 c)5KN/m3 d)8KN/m3

7.In  which method the structural elements are designed by considering load is 		[           ]

a)working stress method	                           b)limit state method

c)ultimate load method 	                                  d)probability load method

8.Permissible stress in concrete is						[           ]

a)0.48 fck           b)0.45 fck           c)0.8 fck               d)0.45 fck

9.Factor of safety in limit state method for steel is						[            ]

a)1.15        b)2.1            c)1.3               d)1

10.For steel, the permissible  stress is 						[             ]

a)0.87 fy          b)0.45fy            c)0.8fck            d)0.45 fck

11. If Xa<Xc, the section is						[            ]

a)under reinforced             b)over reinforced          c)balanced section       d)none

12. Factor of  safety in working stress method  for   steel   is				[           ]

a)1.74        b)2.1           c)1.8          d)1.2

13.Steel  reinforcement in both the tension and compression zone is called                           [           ]

a)single reinforced beam                          b)doubly reinforced beam

c)simply supported beam                          d) none

14.The limiting depth of neutral axis (XUmax /d)for Fe415 steel is                                                    [            ]

a)0.48        b)0.53        c)0.46         d)0.36

15.  Young’s  modulus of steel=						   [            ]

a) 2×105N/mm2 b)2.1×105N/mm2 c)3×105N/mm2 d)3.1×105N/mm2

16.The flange provides to the section 						[            ]

a)resistance b)shear c)tensile resistance d)compressive resistance

17.Breadth of the web is the width of the web portion of the beam in the		[          ]

a)Compression zone b)tension zone c)neutral axis d)none

18.Breadth of the  web is sufficient enough to accommodate the			[            ]

a)concrete b)compression steel c)tension steel d)all the above

19.Breadth of the web is generally of the depth of the web				[           ]

a)1/2  or 2/3 b)1/4 or 2/3 c)1/3 or 2/3 d)1/2 or 1/3

20.The effective depth may vary from L/12 to L/15 for based on the stiffness    		[           ]

a)simply supported beams b)continuous beams c)cantilever beams d)over  hanging beams

Paper Set by K.Mounika

Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

III B.TECH, I SEMISTER MID-1

DESIGN&DRAWING OF REINFORCED CONCRETE SRUCTURES

Write any three questions     							3×10=30

1.Design a RC beam subjected to a bending moment of  20KN/m. Use  M15  concrete and mield steel reinforcement keep the width of the beam is equal to half of the effective depth.

a) Design the above beam M20  grade concrete and Fe250.

b) Design the above beam M15  grade concrete and Fe415  steel.

2. A simply reinforced beam 200mm wide is 400mm deep to the centre of the tensile reinforcement. Determine the limiting moment of resistance of the beam section and also the limit area of reinforcement use M20 	and Fe250 steel.

3. Find the reinforcement required for doubly reinforced sections to the following perticulars width of the beam 250mm and effective depth 500mm to the centre of the reinforcement effective cover to the centre of compression reinforcement 50mm. The maximum moment and working loads 160KN/m use M20  and  Fe250

4. A RCC beam 230mm wide and 450mmdeep is reinforced with 4 bars of 16mm dia and grade Fe415 On tension side. If design shear force is 60KN design the shear reinforcement considering only of vertical stirrups the grade of concrete used is M20

5.A RCC singly reinforced T-beam has a  flange of 750mm width and 120mm thickness is having Ast is 3500mm2 provide at effective depth of 450mm and width of web is 250mm. Calculate the Mu  Of the section using M20 and Fe415

Paper Set by K.Mounika