Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

III B.TECH, I SEMISTER MID-II

WATER RESOURCES ENGINEERING-1

Write any three questions                                                                                                                           3x10=30

1.  A) Explain the various types of irrigation.

b) A water course has a culturable commanded area of 2400 hectares, out of which the intensities of irrigation for perennial sugar cane and rice crops are 22% and 40% respectively. The duties for these crops at the head of water course are 740 hectares/cumecs and 1750 hectares/cumecs respectively. Find the discharge required at the head of water course, if the peak demand is 20% of the average requirement.

2.    a)	Give the effects of water logging.

b)	Give the advantages and disadvantages of lining.

3) Explain briefly Bligh’s creep theory with limitations and also derive the design procedure for Bligh’s creep theory.

4) What Using Kennedy’s theory design a channel section with the following data.

Discharge=16cumecs, N= 0.0225, m=1, side slope= ½:1, bed slope = 1/5500 and

5) Explain the various components of Diversion head work with neat sketch ?

Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

III B.TECH, I SEMISTER MID-II

WATER RESOURCES ENGINEERING-I

objective                                                                                       20x1/2=10

1) Irrigation is defined as……………supply of water                                  [       ]

a)artificial             b)natural	c)both a&b		d)none

2) In India ………irrigation is mostly adopted for irrigating the crops           [         ]

a)sub surface		b)surface	c)sprinkler	d)none

3) ………………….irrigation is also known as over head irrigation            [         ]

a)sub surface		b)surface	c)sprinkler	d)none

4) ……is defined as the total depth of water required by the crop during the complete base perio[	]

a)duty	b)capillary depth	c)saturated depth	d)delta

5) the water exists in the pore space of the soil by surface tension is known as ……….	[	]

a) Capillary water	b) permeable water           c) fissured water        d) none

6) The units of duty is…………                                    [	]

A) hectares	b) hectares/cumecs	c) hectares/meters	d) none

7) The relationship between duty(d),delta(Δ) and base period (B) IS			[	]

a)D=0.64Δ/B	b) D=8.64Δ/B	c) D=8.64B/Δ	d)NONE

8) The base period of a particular crop is 100days.if the duty is 400 Hec/cumes the delta required for    crop is                      [	]

a) 6.17   	b)617		c)61              d) 61.7

9) The sprinkler  irrigation system is suitable , when	[	]

a) land gradient is steep                             b) crop should have deep roots

c)soil has low permeability                          d) water table is low

10for design of a lined canal the commonly used formula is    [         ]

a) lacey’s formula           b) Kennedy’s formula		c) manning’s formula	d)none

11) As per BIS the shape of the lined canal is…………….        [   	]

a)rectangle	b)parabolic	c) trapezoid           d)none

12) ………is provided in to safeguard the floor against piping for soil consideration [ 	]

a)lining	b)head regulator	c) canal regulator	`d)existing gradient

13) the depth of water consumed by crop during the process of evaporation and transpiration of crop growth is known as …………….[    	]

a)consumptive use	b)head regulator	c) canal regulator	`d)existing gradient

14) the stable channel which transports a regime silt charge is called…….. channel[	]

a)lacey’s channel	b)regime channel	c)Kennedy’s channel	d)none

15) the structure constructed across the river to divert the water to the canal is called	[	]

a) weir  	b) barrage	c)head work   d)none

16) the canal which carries water for hydro electric  power is called……[	]

a)navigation canal	b)feeder canal	c)carrier canal	d)power canal

17) feeder canal is also called as           [	]

a)link canal	B) feeder canal	c)carrier canal	d)power canal

18) canal which act as a permanent source is called ……. Canal    [	     ]

a)temporary              b)permanent		c)both a&b	d)none

19) ……….is defined as a structure provided at head of water course for connecting it to the distributory channel  [	]

a) canal outlet		b)canal diversion		c) canal regulator 		d) canal head 20) diversion head works are constructed usually  across ……. rivers      [        ]

a)perennial		b)inundation		c)both a&b	d)none

Paper set by G.Sowjanya