Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

III B.TECH, I SEMISTER MID-II

WATER RESOURCES ENGINEERING-I

objective                                                                                       20x1/2=10

1) Irrigation is defined as……………supply of water                                  [       ]

a)artificial             b)natural	c)both a&b		d)none

2) In India ………irrigation is mostly adopted for irrigating the crops           [         ]

a)sub surface		b)surface	c)sprinkler	d)none

3) ………………….irrigation is also known as over head irrigation            [         ]

a)sub surface		b)surface	c)sprinkler	d)none

4) ……is defined as the total depth of water required by the crop during the complete base period[ 	]

a)duty	b)capillary depth	c)saturated depth	d)delta

5) the water exists in the pore space of the soil by surface tension is known as ……….	[	]

a) Capillary water	b) permeable water           c) fissured water        d) none

6) The units of duty is…………                                    [	]

A) hectares	b) hectares/cumecs	c) hectares/meters	d) none

7) The relationship between duty(d),delta(Δ) and base period (B) IS			[	]

a)D=0.64Δ/B	b) D=8.64Δ/B	c) D=8.64B/Δ	d)NONE

8) The base period of a particular crop is 100days.if the duty is 400 Hec/cumes the delta required for    crop is                      [	]

a) 6.17   	b)617		c)61              d) 61.7

9) The sprinkler  irrigation system is suitable , when	[	]

a) land gradient is steep                             b) crop should have deep roots

c)soil has low permeability                          d) water table is low

10for design of a lined canal the commonly used formula is    [         ]

a) lacey’s formula           b) Kennedy’s formula		c) manning’s formula	d)none

11) As per BIS the shape of the lined canal is…………….        [   	]

a)rectangle	b)parabolic	c) trapezoid           d)none

12) ………is provided in to safeguard the floor against piping for soil consideration [ 	]

a)lining	b)head regulator	c) canal regulator	`d)existing gradient

13) the depth of water consumed by crop during the process of evaporation and transpiration of crop growth is known as …………….[    	]

a)consumptive use	b)head regulator	c) canal regulator	`d)existing gradient

14) the stable channel which transports a regime silt charge is called…….. channel[	]

a)lacey’s channel	b)regime channel	c)Kennedy’s channel	d)none

15) the structure constructed across the river to divert the water to the canal is called	[	]

a) weir  	b) barrage	c)head work   d)none

16) the canal which carries water for hydro electric  power is called……[	]

a)navigation canal	b)feeder canal	c)carrier canal	d)power canal

17) feeder canal is also called as           [	]

a)link canal	B) feeder canal	c)carrier canal	d)power canal

18) canal which act as a permanent source is called ……. Canal    [	     ]

a)temporary              b)permanent		c)both a&b	d)none

19) ……….is defined as a structure provided at head of water course for connecting it to the distributory channel  [	]

a) canal outlet		b)canal diversion		c) canal regulator 		d) canal head 20) diversion head works are constructed usually  across ……. rivers      [        ]

a)perennial		b)inundation		c)both a&b	d)none

Paper set by G.Sowjanya