Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

II B.TECH, I SEMESTER

FLUID MECHANICS

Time: 3 hours                                                                                                                       Max. Marks: 70

PART-A

(Compulsory Question)

1)          Answer the following: (10X2=20 Marks)

- What is Pascal’s law?

- Define buoyancy?

- How does Pitot tube works?

- Define and differentiate absolute and gauge pressures

- What is a flownet?

- What is vena-contracta and where does it occurs?

- How does viscosity varies with temperature for liquids and gases?

- What is energy correction factor?

- What is an orifice and define mouthpiece.

- Define velocity of approach.

PART-B

(Answer all five units, 5X10=50 Marks )

UNIT-I

2)    a)	Define surface tension and viscosity of a fluid.

b)	What are Newtonian and Non-Newtonian fluids

OR

3)	Define total pressure and centre of pressure. Also derive the expressions for total pressure and centre of pressure for a plane surface which is immersed in water at an angle Ѳ with free water surface.

UNIT-II

4)     a)	Define meta-centre and meta-centric height .

b)	A block of wood of specific gravity 0.7 floats in water. Determine the meta-centric height of the block if its size is 2m X 1m X 0.8m.

OR

5)     a)	A stream function in a two dimensional flow is Ψ = 2xy. Show that the flow is irrotational and determine the corresponding velocity potential ф.

b)	An open circular cylinder of 15cm diameter and 100cm long contains water at a height of 50cm. find the maximum speed at which the cylinder is to be rotated about its vertical axis, so the water spills.

UNIT-III

6)     a)	Give the necessity of the energy correction factor and derive the expression for the same.

b)	What is momentum principle. Explain its application to pipe bend.

OR

7)	What is venturimeter? How does it works? Derive the expression for rate of flow of a venturimeter.

UNIT-IV

8)	A reservoir discharges through a sluice 0.915m wide by 1.22m depth. The top of the opening is 0.61m below the water level in the reservoir and the downstream water level is below the bottom of the opening. Calculate the discharge through the opening  if Cd=0.60 and percentage error of the opening is treated as a small orifice.

OR

9)	What is a notch? Give the classification of notches.

UNIT-V

10)   a)	List out the various minor losses that may occur when the fluid is flowing through a pipeline and also give formulae to calculate the losses.

b)	A pipe 50mm diameter is 6m long and the velocity of flow of water in the pipe is 2.4m/s. what is the loss of head and the corresponding power would be saved if the central 2m length of pipe was replaced by 75mm diameter pipe, change of the section being sudden? Take f= 0.04 for both the pipes

OR

11)   a)	Explain Reynold’s experiment.

b)	Define equivalent pipe and also derive the corresponding equation.