Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

II B.TECH, I SEMESTER

STRENGTH OF MATERIALS-I

Time: 3 hours                                                                                                                       Max. Marks: 70

PART-A

(Compulsory Question)

1)          Answer the following: (10X2=20 Marks)

- Explain Elastic limit?

- Explain Resilience and sudden loading?

- Explain Cantilever beam and its end conditions?

- Explain point of contra flexure?

- What are the assumptions made in deriving the bending equation?

- Define section modulus for circular section?

- Define Slope, deflection and Radius of curvature?

- Explain moment area method?

- Explain conjugate and real beam?

- Give example for finding slope and deflection using conjugate beam method?

PART-B

(Answer all five units, 5X10=50 )

UNIT-I

2) A tensile load of 50KN is acting on a rod of diameter 35mm and length of 6m. A bore of       diameter 25mm is made centrally on the rod. To what length the rod should be bore, so that the total extension will increase 30% under the same tensile load. Take E= 2x105 N/mm2.

Or

3) A metallic bar 300mm X 100mm X 50mm is subjected t9o a force of 6KN (tensile), 8KN (tensile) and 5KN (tensile) along X, Y, Z directions respectively. Determine the change in the volume of the block. Take E = 2 X 105 N/mm2 and poisson’s ratio 0.25.

UNIT-II

4) A Simply supported beam of length 6m carries a uniformly increasing load of 600 N/m at one end to 1500 N/m run the other end. Draw Shear Force and Bending Moment diagram for the beam. And also calculate the position and magnitude of maximum Bending moment.

Or

5) A Cantilever of length 3m carrying a uniformly distributed load of 1.5KN/m run over a length of 2m from the free end. Draw SFD and BMD.

UNIT-III

6) Derive bending equation.

Or

7) A beam of triangular cross section with base b and height h is used with the base horizontal. Determine the maximum shear stress and plot the variation of shear stress intensity along the section.

UNIT-IV

8) Derive the relation between slope, deflection and radius of curvature.

Or

9) A beam of length 8m is simply supported at its end and carries two point loads of 36KN and 40KN at a distance of 1.5m and 4m from the left support. Find i) deflection under each load.

ii) Maximum deflection and

iii) the point at which maximum deflection occurs, given E= 2x105 N/mm2 and I= 85x106 mm4.

Using Macaulay’s method.

UNIT-V

10) A hollow rectangular column of external depth 1.5m and external width.8m is 12cm thick. Calculate the maximum and minimum stress in the section of the column if a vertical load of 220KN is acting with an eccentricity of 16cm.

Or

11) A simply supported beam of length 5m carries a point load of 2.5KN at a distance of 1m from each end. If E= 2x105 N/mm2 and I= 1x108 mm4 for the beam, then using conjugate beam method. Determine i) Slope at each end under each load

ii) Deflection under each load and at the centre.