Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

II B.TECH, I SEMESTER GRAND TEST

SURVEYING-1

Time: 3 hours                                                                                 Max. Marks: 70

PART – A

(Compulsory Question)

1)          Answer the following: (10X2=20 Marks)

(a) What is the object or purpose of surveying?

(b) Define chain surveying. What is the fundamental principle of chain surveying?

(c)What do you understand by whole circle bearing and quardantal bearing of a line?

(d) Name the errors in plane tabling.

(e)What are the different kinds of bench marks?

(f)What are the different characteristics of contour?

(g)List the essential parts of a Theodolite.

(h) Name the two methods of measuring horizontal angles using a theodolite.When each

Method is advantageously used?

(i) What is Simpson’s rule? What are the limitations of this method?

(j) What is prismoidal rule?

PART-B

(Answer all five units, 5X10=50 Marks)

UNIT-I

2     Discuss about the classification of surveying.

(a) Based on object of survey.

(b) Based on instruments employed.

OR

3   (a) What factors should be considered in deciding the station of a chain survey?

(b) What is local attraction? How is it detected and eliminated?

UNIT-II

4   The following were observed in a compass traverse. Correct for local attraction.

| Line | Fore bearing | Back bearing |
| --- | --- | --- |
| AB | 44030’ | 226045’ |
| BC | 124030’ | 303015’ |
| CD | 181000’ | 1000’ |
| DA | 289030’ | 108030’ |

OR

5    List out methods of plane tabling and explain any one method with a neat sketch

UNIT-III

6   The following staff readings were observed with a level and a 4m staff on a continuously sloping ground  at a common interval of a 15m: 0.880,1.635,2.055,2.530,3.085,3.580,1.255,2.060,2.465,3.740,1.145,1.730 and 2.645.The reduced level of the first point was 780.150.Rule out a page of a level-book and enter above readings. Calculate the reduced levels and the gradient of the line joining the first and last point.

OR

7    Explain about the characteristics of contours with neat sketches.

UNIT-IV

8    What are the different errors in Theodolite work? How are they eliminated?

OR

9   Define the following terms in Theodolite:

(a)Vertical axis (b) Horizontal axis (c) Centering  (d) Transiting (e)Changing face

UNIT-V

10  (a)  The perpendicular offsets taken at 10m intervals from a survey line to an irregular boundary are 2.18m,3.3m ,4.26m,6.2m,4.8m,7.20m,8.8m,8.2m and 5.2m.determine the area enclosed between the boundary, survey line, the first and last offsets by (i)Trapezoidal rule (ii) Simpson’s rule.

(b)  Find out the volume of earth work in a road cutting 120m long from the following data. The formation width 10 meters , side slopes 1 to 1; average depth of cutting along the centre of line 5m, slope of ground in cross-section  10 to 1

OR

11   The following offsets were taken from a chain line to a hedge

| Chain(m): | 0 | 15 | 30 | 45 | 60 | 70 | 80 | 100 | 120 | 140 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Offset(m): | 7.60 | 8.5 | 10.7 | 12.8 | 10.6 | 9.5 | 8.3 | 7.9 | 6.4 | 4.4 |

Compute the area by

(a) Trapezoidal rule (b) Simpson’s rule (c) Average ordinate rule.