Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

III B.TECH, I SEMESTER GRAND TEST

BUILDING PLANNING AND DRAWING

Time: 3 hours                                                                                 Max. Marks: 70

PART – A

(Answer the following: 03 X 10 = 30 Marks)

UNIT-1

1. (a) What are building bye laws? Explain them briefly.

(b)Write short notes on Floor Area Ratio (FAR), how it is related to height of the building. Explain.

OR

2. (a) Explain in detail the factors to be considered for selection of a site for a residential building?

(b)On what considerations, the grouping of various units in residential buildings is made?

UNIT-2

3. Explain the characteristics of various types of residential buildings, with sketches.

OR

4(a) what are the minimum standards of various parts of Hospital and Dispensaries?

(b) Discuss the principles of planning of school building.

UNIT – 3

5. Distinguish between PERT and CPM in detail.

OR

6. From the utility data for a network are given below. Determine the total, free, independent and interfering floats and identify the critical path.

| ACTIVITY | 0-1 | 1-2 | 1-3 | 2-4 | 2-5 | 3-4 | 3-6 | 4-7 | 5-7 | 6-7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DURATION(in days) | 2 | 8 | 10 | 6 | 3 | 3 | 7 | 5 | 2 | 8 |

PART – B

(Answer the following: 01 X 10 = 10 Marks)

UNIT –4

7. Draw to a suitable scale the front elevation of a King post truss indicating all details for a clear opening of 8000 mm.

OR

8. Draw conventional signs for the following materials:

(i) Brick. (ii) Concrete. (iii) Wood. (iv) Stone. (v) Earth

UNIT-5.

(Answer the following: 01 X 30 = 30 Marks)

9. The line sketch of the plan of a residential building is shown in figure below. Draw:

(a) A neat dimensioned plan.

(b) Sectional elevation along AB, to a suitable scale, using the following specification.

Specifications: Foundations: C.C 1:4:8 800 mm wide and 300 mm thick.

Footings: Rubble stone masonry: 600 mm x 500 mm.

Basement: Coursed rubble masonry: 400 mm wide and 700 mm high.

Superstructure: Brickwork in C.M 1.5:300 mm wide and 300 mm high.

R.O.C roofing: 100mm thick.

OR

10.Figure shown in below the line drawing of a residential building, draw to a scale of the following:

(a) Plan. (b) Section along AB. (c) Front elevation.

The following specifications are to be adopted:

Foundation: Depth 1000 mm. C.C bed 1000 mm x 300 m

Two footings with an off set of 50 mm and 250 mm thickness each.

Basement: 600 mm high, thickness of wall at this level is 400 mm.

Walls : Brick masonry in C.M.1:6, 300mm thick

Roof: R.C.C slab 120mm thick.

Provide the details of doors, windows, ventilators and steps etc. as per standard dimensions