Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

III B.TECH, I SEMESTER

DESIGN & DRAWING OF REINFORCED CONCRETE STRUCTURES

Time: 3 hours                                                                                                                       Max. Marks: 70

Use of IS 456:2000, SP16 Design aided charts only and IS: 875 (Part-1 & Part-2) books are permitted in the examination hall.							PART-A

(Answer any one question: 01 x 28 = 28 Marks)

1. 	Design an R.C.C. slab of size 5 m x 6 m, simply supported on all four edges with

corners held down. The slab is carrying a load of 4 KN/m2 including floor finish etc.

Use M 20 concrete and Fe 415 steel. Sketch the details of reinforcement also. 28m

Or

2. 	A rectangular reinforced concrete beam is simply supported on two masonry walls

230 mm thick and 6 m apart (centre to centre). The beam is carrying an imposed

load of 15 KN/m. Design the beam with all necessary checks. Use M25 concrete

and Fe 415 steel. Sketch the details of reinforcement.

PART-B

(Answer any one question: 03 x 14 = 28 Marks)

3. a) 	Draw stress-strain relationship for concrete and explain it briefly.

b) 	Find the moment of resistance of a beam section 250 mm x 500 mm deep is

reinforced with 2- 16 mm bars in tension at an effective cover of 40 mm. Use M20

concrete and Fe 500 grade of steel.

4. 	A simply supported R.C.C. beam 200 mm x 400 mm (effective) is reinforced with

4 bars of 22 mm diameter on tension side. The beam is carrying a load of 10 KN/m

over a clear span of 8 m. Design the shear reinforcement. Use M 20 concrete and

Fe 415 steel bars.

5. 	Design the footing for a reinforced concrete column 225 x 450 mm carrying an axial

load of 1075 KN. The bearing capacity of the soil is 100 KN/m2. Use M20 concrete

and Fe500 grade steel as reinforcement.

6. 	Design the waist slab type stair case consisting of a straight flight of stairs resting

on two stringer beams along the two sides. Assume the span of the slab as 2 m with

risers of 160 mm and treads of 270 mm. Live load= 3 KN/m2. Adopt M20 grade

concrete and Fe 415 grade steel.

7. 	Design a R.C.C. column to carry an axial load of 2000N. The size of the column is

restricted to 600 mm square. The effective height of column is 9 m. Use M20

concrete and fsc = 190 N/mm2.