Gokula Krishna college of Engineering –Sullurpet                                                               Department of CIVIL Engineering                                                                                                              III B.TECH, I SEMESTER                                                                                                                                      STRUCTURAL ANALYSIS-II

Time: 3 hours                                                                                                                       Max. Marks: 70

PART-A

(Compulsory Question)

1)          Answer the following: (10X2=20 Marks)

- Distinguish between pin jointed and rigid jointed structures?

- Differentiate the statically determinate structure and statically indeterminate structure?

- Under what conditions will the bending moment in an arch be zero throughout?

- Explain rid-shortening in the case of arches?

- What are the advantages of continuous beam over simply supported beam?

- Mention the situation where in sway will occur in portal frames?

- What are the assumptions made in slope-deflection method?

- Give the assumptions for fully plastic moment of a section?

- A T-section consists of 20mm web and 20mm thick flange. Depth of the web is 180mm. width of the flange is 120mm. find the shape factor based on plastic analysis?

- A rigid frame is having totally 10 joints including support joints. Out of slope-deflection and moment distribution methods, which method would you prefer for analysis? Why?

PART-B

(Answer all five units, 5X10=50 )

UNIT-I

2) A three hinged parabolic arch has a span of 60m and central rise of 8m. it is subjected to a point load of 40KN at a distance of 10m from left support and a UDL of 10KN/m over right half span. Calculate the location and magnitude of maximum bending moment. Also calculate the radial shear, normal thrust and bending moment under 40KN load.

Or

3) A three hinged parabolic arch of span 40m and rise 8m carries concentrated loads of 200KN and 150KN at distance of 8m and 16m from left end and an UDL of 50KN/m on the right half of the span. Determine the horizontal thrust. Also calculate the radial shear, normal thrust and bending moment at a distance of 8m from left end.

UNIT-II

4) Analyze the continuous beam shown in figure below, using slope deflection method. Draw shear force and bending moment diagram for the continuous beam.

Or

5) Analyze the continuous beam shown in figure below, using moment distribution method. Draw shear force and bending moment diagram for the continuous beam.

UNIT-III

6) Using Kani’s method, analyze the portal frame shown in figure below.

Or

7) Using Kani’s method, analyze the continuous shown in figure below.

UNIT-IV

8) Analyze the beam given in below, by flexibility method. Take EI constant. The beam undergoes settlement of supports B and C by 10/EI and 5/EI respectively. Draw the bending moment diagram.

Or

9) Analyze the continuous beam in given below, by stiffness method diagram. Take AB=2I, BC=CD=I.

UNIT-V

10) Find the required value of plastic moment capacity at collapse load for the continuous beam shown in figure below. Take load factor as 2. Take AB=2Mp; BC=1.5Mp; CD=Mp

Or

11) Determine the plastic moment capacity of a continuous beam shown in figure below. Take λ=1.7.