Gokula Krishna college of Engineering –Sullurpet

Department of CIVIL Engineering

III B.TECH, I SEMESTER

WATER RESOURCES ENGINEERING-I

Time: 3 hours                                                                                                                       Max. Marks: 70

PART-A

(Compulsory Question)

1)          Answer the following: (10X2=20 Marks)

- List out various types of precipitation.

- Define hydrological cycle.

- If the following are the ordinates of a 2hr unit hydrograph, what will be the ordinates of 2hr hydrograph due to the excess rainfall of 3cm?

| Time(hr) | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2hr U.H (M3/s) | 0 | 6.4 | 12.2 | 16.4 | 17.3 | 15.8 | 8.5 | 3.7 | 0 |

- If specific yield of an aquifer is 17.4% and its specific retention is 15.4%. what would be its porosity?

- Give the necessity of lining of canals.

- What is Regime canal?

- Define runoff.

- Define irrigation and also delta and duty.

- What is meant by evaporation and transpiration?

- Define kor depth and kor period.

PART-B

(Answer all five units, 5X10=50 Marks)

UNIT-I

2)    a)	What are the various forms of precipitation? Explain each.

b)	Explain Isohyetal method and Theisson polygon method with suitable example.

Or

3)    a)	What are the various factors affecting evaporation.

b)	List out various methods with which we can measure the evaporation.

UNIT-II

4)    a)	Describe the assumptions and the construction of a Unit Hydrograph.

b)	Derive the dupuits theory for confined and unconfined aquifers

Or

5)   a)	Explain S-Hydrograph.

b)	Explain the terms specific yield, specific retention and their relation in terms of porosity.

UNIT-III

6)   a)	What are the advantages and disadvantages of irrigation.

b)	Explain the various types of irrigation.

Or

7)	A water course has a culturable commanded area of 2400 hectares, out of which the intensities of irrigation for perennial sugar cane and rice crops are 22% and 40% respectively. The duties for these crops at the head of water course are 740 hectares/cumecs and 1750 hectares/cumecs respectively. Find the discharge required at the head of water course, if the peak demand is 20% of the average requirement.

UNIT-IV

8)	Using Kennedy’s theory design a channel section with the following data.

Discharge=16cumecs, N= 0.0225, m=1, side slope= ½:1, bed slope = 1/5500 and

Discharge= 10cumecs, N= 0.225, m=1, B/D=3.5 (do both the problems)

Or

9)     a)	Give the effects of water logging.

b)	Give the advantages and disadvantages of lining.

UNIT-IV

10)   	Explain the various components of diversion head work with neat sketch.

Or

11)	Explain briefly Bligh’s creep theory with limitations and also derive the design procedure for Bligh’s creep theory.

Paper set by G. SOWJANYA